Texas Instruments

Keystone II Multicore Workshop

ARM-based Lab Manual

February 2014

Contents

Prerequisites	1

Hardware	1

Software	1

Workshop Network	2

Setting up a Serial Terminal Session to the EVM via USB	3

Setting up a VNC View to the Ubuntu Server	3

Lab 1 – EVM Board Bring-up and OOB Demonstration	1

Purpose	1

Task 1: Loading the U-BOOT	1

Verify the Current U-BOOT	1

Program SPI NOR Flash with U-boot GPH image	2

Task 2: Load and Run standard “Hello World” application	3

Lab 2 – Build a New ARM Program	9

Projects and source code	9

Purpose	9

Task 1: Modify the File system	9

Example Simple Code	10

Build the Executable	10

Unzip and Decompress the File System and add the new executable	11

Compress and Zip the New File System	12

Reboot the EVM and Run the New Program	12

Task 2 (optional): Build U-boot, Boot Monitor and Kernel	13

U-BOOT Source Code Extraction	13

U-BOOT Build Instructions	14

Boot Monitor (skern.bin) Source Code Extraction and Build Instructions	15

Linux Kernel and Device Tree  Source Code Extraction and Build Instructions	16

Lab 3: Boot Using NFS-mounted File System	19

Purpose	19

Task 1: Build a file system on a Linux Host, Use the NFS server	19

Task 2: Configure U-BOOT to mount the file server and boot	20

Task 3: Build a new C program in the file system, and debug it	21

Lab 4: Build, Run and Optimize DSP Project using CCS	23

Purpose	23

Project Files	23

Task 1: Build and Run the Project	23

Task 2: Define the Target	31

Task 3: Connect to the EVM	32

Task 4: Load and Run CASE 1	33

Task 5: Use Optimization and Disable Symbol Debug for the fir Filter Routine	34

Task 6: Optimize Software Pipeline	36

Task 7: Enable the Cache	38

Task 8: Running in Parallel on Multiple Cores	39

Lab 5: Load and Run DSP Code Using MPM Server	43

Purpose	43

Project Files	43

Task 1: Build and Run the Project	43

Task 2: Using MPM to load, run and observe results	46

Lab 6: ARM-DSP Communication Using MPM & Shared DDR	47

Purpose	47

Linux and DSP simple memory management	47

Project Files	50

Task 1: Build and Run the Project	50

Lab 7: Build and Run ARM Code Using CCS on Linux	53

Purpose	53

Prerequisites	53

Task 1: CCS - Start a new cross compiler project and build it	53

Task 2: GDB from the tera-terminal	59

Move the Debug directory outside of CCS	59

Task 3: Connect a SSH terminal into the target	60

Task 4: Optional – Use gdb to debug the application	70

Task 5: Configure the host proxy	74

# Prerequisites

The following hardware and software are needed to perform the labs in this manual.

## Hardware

- Update BMC and UCD on EVMK2H (optional):

  - The wiki page http://processors.wiki.ti.com/index.php/EVMK2H_Hardware_Setup gives instructions on how to detect if the board needs BMC (Baseboard Management Controller) update. It also instructs how to do the update the BCM using CCS.
NOTE: A PDF version of the wiki page (KeyStone2_EVM_hardwareSetUp.pdf) is also available. Ask your instructor.

  - The user must also check the UCD Power Management version (see EVMK2H Hardware Setup at link above) and update if necessary.
NOTE: Instructions and scripts that show how to update the UCD are provided in the zip file XTCIEVMK2X-UCD-Update.zip (ask your instructor).

## Software

The following software packages must be pre-installed on the student laptop before the workshop starts. Note: During the workshop the laptop is attached to local network and has limited access to internet.

- Download the MCSDK and CCS

  - For details regarding the instructions in this section, refer to the MCSDK User Guide for KeyStone II.

  - The latest release of MCSDK is found here:
http://software-dl.ti.com/sdoemb/sdoemb_public_sw/mcsdk/latest/index_FDS.html

    - For this lab you can use the Windows or the Linux version, depends on your laptop. Linux MCSDK was pre-installed on an Ubuntu server that will be used in some of the labs.

  - From the same download page as the MCSDK, locate and download the latest CCS version and the emupack version that goes with the CCS. Follow the instructions on the page. Note, installing CCS requires licensing from TI.

- Installing VNC Viewer

VNC server that supports graphic interface was installed on the Ubuntu server. Each laptop must have a VNC viewer.  Texas Instruments and many other corporations purchased global licenses for Real VNC enterprise users and it can be downloaded from internal software download site (EDS). Limited functionality Real VNC viewer is available as freeware from multiple sites.

- FTP Client

FTP server is installed on the Ubuntu server. Moving files between the student Laptop and the Ubuntu server can be done with the enterprise version of Real VNC or (if the student uses a freeware real VNC) by using ftp client on the laptop. The student must confirm that ftp client is installed on the laptop.

- For communication between the student PC and the EVM, the FTDI driver is required. As needed, download the 32-bit driver here: http://www.ftdichip.com/Drivers/D2XX.htm

### Workshop Network

The diagram above shows the workshop network environment:

- There are up to 11 lab stations. EVMs at each station are numbered from 1 to 11. Each station has the following:

  - One EVMK2H

  - One laptop that is connected to the EVM via JTAG cable, called the first laptop.

  - One optional laptop that is not connected to the EVM, called the second laptop in the station.

- All EVMs and students laptops are connected to the local network 192.168.0.XX via a wired connection to a switch or a router.

- The Ubuntu server is connected as well. The Ubuntu server (or a router) has access to an external network with a global IP that have access to the Web.  The IP address may be given by DHCP server on the Ubuntu or DHCP server on a router.

### Setting up a Serial Terminal Session to the EVM via USB

The EVM has two (mini) USB ports.

  - One of the ports accesses the JTAG connection and can be used to connect CCS to the board. This USB connector is part of the emulator daughter (mezzanine) card.

  - The second USB port is part of the mother board and can be used to connect two serial terminals into the EVM. We will refer to these serial terminals as Tera-Terminal (to distinguish from the window viewer terminal to the Ubuntu machine).  The tera-terminals are connected using a single USB cable but can be opened as two tera-terminals.

    - The first serial terminal (ARM Tera-terminal) is connected to the ARM terminal (e.g., the lower COM port)

    - The second serial terminal (BMC Tera-terminal) is connected to the FPGA/BMC on the board.

    - The user must open the two tera-terminals and set the serial rate to 115200 Baud.

### Setting up a VNC View to the Ubuntu Server

- Launch the VNC Viewer application from the desktop of your laptop/PC.

- The server IP will be given by the instructor. For static configuration, when DHCP is not available, the server IP is 192.168.0.100.

- The login instance for student N is :N. For example, student 3 will VNC to address 192.168.0.100:3, while student number 7 will use 192.168.0.100:7.

- The VNC password for all students is “vncserve ”

# Lab 1 – EVM Board Bring-up and OOB Demonstration

## Purpose

The purpose of this lab is to complete the out-of-box (OOB) demonstration that shows how to boot and run a very basic “hello world” program using u-boot. You will load the kernel and file server and run a pre-built hello world

## Task 1: Loading the U-BOOT

The U-BOOT that is programmed into flash on the EVM may need to be updated when moving to the most recent version of the MCSDK. First, determine which version of U-BOOT is currently programmed  into flash. Second, load the current U-BOOT image into SPI NOR flash, as needed.

### Verify the Current U-BOOT

To verify if the U-BOOT is already loaded, perform the following steps:

- Set Dip Switch (SW1) on the EVM to 1 OFF  2 OFF  3 ON  4 OFF

- Open two terminals into the EVM using a single USB cable connected to the lower USB connection on the board. From serial port setting chose Baud rate of 115200

- Power up the EVM.  If the U-BOOT is already burned into the flash, the terminal will look like the following:

U-Boot SPL 2013.01 (Apr 05 2013 - 14:12:32)

SF: Detected N25Q128A with page size 64 KiB, total 16 MiB

U-Boot 2013.01 (Apr 05 2013 - 14:12:32)

I2C:   ready

DRAM:  1 GiB

WARNING: Caches not enabled

NAND:  512 MiB

Net:   Ethernet PHY: 88E1111 @ 0x00

TCI6614-EMAC

Warning: failed to set MAC address

Hit any key to stop autoboot:  0

### Program SPI NOR Flash with U-boot GPH image

The following process is used to update the U-BOOT image in SPI Flash.

- Power cycle the EVM and stop the autoboot by pressing any key.

- Copy u-boot-spi.gph from the images in the MCSDK release to the tftp server root directory.

NOTE: When using the standard training server, u-boot-spi.gph should be copied to directory /var/lib/tftpboot/studentN where N is the student number. Then add path to studentN, e.g., dhcp 0xc300000 studentN/u-boot-spi-keystone-evm.gph

- Rename the image file to u-boot-spi-keystone-evm.gph

- Make sure the tftp server is running. Then issue the following commands to U-Boot console:

setenv serverip <your tftp address>

dhcp 0xc300000 u-boot-spi-keystone-evm.gph

sf probe

sf erase 0 <size of u-boot-spi.gph in hex up rounded to sector boundary of 0x10000>

sf write 0xc300000 0 <size of u-boot-spi.gph image in hex>

NOTE: The size of the image will be displayed as part of the DHCP command.

NOTE: If u-boot-spi.gph was not copied to tftp root directory, then specify the path

e.g.: dhcp 0xc300000 release/u-boot-spi-keystone-evm.gph

### Task 2: Load and Run standard “Hello World” application

- The base location of the tftp server is defined in the tftp configuration file (/etc/xinetd.d/tftp) as the server_args.

- The default setting is /var/lib/tftpboot/studentN where student is the user name (N is 1, 2, 3, …10).

- In order for the U-BOOT to get files from sub-directory, the subdirectory should be the tftp_root value. For example, if all the files that are needed for U-BOOT are in directory /var/lib/tftpboot/student, then (see below) the tftp_root value of the U_BOOT is studentN

- Make a subdirectory /var/lib/tftpboot/studentN if it does not exist already  and copy the MCSDK release binary images into this directory. The binary images are located in the following directory on the Ubuntu server:

/opt/ti/MCSDK_3_XX/mcsdk_linux_3_00_00_xx\images

Wheere XX is the most current MCSDK release number

- You can also move the images from the release of MCSDK on your laptop using FTP. Use the same directory: /var/lib/tftpboot/studentN

- U-BOOT loading and running Linux Kernel using TFTP with ramfs file system

  - First set the DIP switch (sw1) to ARM SPI boot mode:
1 OFF 2 OFF 3 ON 4 OFF (for Rev 1 EVM)

  - Power off EVM

  - Disconnect CCS from EVM (if already connected)

  - Power up EVM , look at the ARM  tera-terminal window

- It is very important to set the environment variables correctly for U-BOOT.  Here are the instructions how to do so:

  - After power cycle, press ESC to stop autoboot in the ARM tera-terminal

  - Enter the following command to show the current environment variables.

env default -f -a

  - If the sizeof file system is larger than 9M bytes, update the value to match it, note the limitation is: 80 MBytes:

setenv args_ramfs 'setenv bootargs ${bootargs} earlyprintk rdinit=/sbin/init rw root=/dev/ram0 initrd=0x802000000,80M'

  - Set the following environment variables:

- setenv name_fs the name of the compressed file system to load. e.g., tisdk-rootfs.cpio.gz (look at the release for an updated name)

- setenv name_fdt  the name of the binary device tree. e.g., uImage-k2hk-evm.dtb (look at the release for an updated name)

- setenv name_kern    the name of the kernel file. e.g., uImage-keystone-evm.bin (look at the release for an updated name)

- setenv name_mon  the name of the kernel monitoring file. e.g., skern-keystone-evm.bin (look at the release for an updated name)

- setenv serverip  IP address of the TFTP server (the Ubuntu server, default IP for workshop is 192.168.0.100)

- setenv boot ramfs

- setenv tftp_root student

- saveenv

  - Boot the EVM using either a hardware or software reboot.

- Hardware reboot = power cycle

- Software reboot = type fullrst in the BMC terminal window.

- The tera-terminal will start as follows:

- When booting ends, login as root (no password)

- Go to directory /usr/bin   cd /usr/bin

- Run the hello program   ./hello

- The program should print hello world on the tera-terminal.

# Lab 2 – Build a New ARM Program

## Projects and source code

Unless instructed by the instructor otherwise, all projects and source code are available on the Ubuntu server. The directory /usr/local/projects has two sub-directories:

- /usr/local/projects/ARM contains ARM projects

- /usr/local/projects/DSP contains DSP projects

NOTE: When the DSP projects are built using CCS on the student PC, projects should be moved via FTP from the server to the student laptop.

## Purpose

The purpose of this lab is to demonstrate how to build and run a simple ARM program using all the development tools on Ubuntu server. You will then load the new file server to the EVM and execute the code.

### Task 1: Modify the File system

First, you will modify the file system that was previously loaded onto the EVM.  A complete build of all other images that are part of the release is an optional exercise. The instructions how to build the U-Boot, boot monitor and the kernel  images are provided in Task 2.

Modifying the file system involves three steps.

- First, a new main function is developed. Using the cross compiler tools on Ubuntu, the function is compiled and an executable is built.

- Next, the current compressed file system is unzipped and de-compressed into a temporary directory, and the new executable that was built in the previous step is added.

- Last, the new file system is compressed, zipped, and moved to the tftp directory. The EVM is then booted, the new program is executed, and produces the expected results.

### Example Simple Code

The instructor will provide you with a simple C program that does elementary calculations and prints out some comments and the results of the calculations.

- For the standard TI workshop, the example code is located on the Ubuntu server in /usr/local/projects/ARM directory. Assume that the example file name is example1.c

NOTE: If the example code name is different, substitute the correct name for example1 throughout the process outlined in this document.

- Copy this file to the student directory /home/studentN/temp

NOTE: If the temp directory does not exist, create it as follows:

- cd ~/      takes you to the home directory

- sudo mkdir temp

NOTE: When sudo is used, the system may prompt for a password.
Password = WsN (where N is your student number)

- /usr/local/studentStartScript.sh is a script that defines all the paths and exports for each individual user. The user must call this script for each new terminal:

source /usr/local/studentStartScript.sh

- The Linaro toolchain and all other shared software are installed on the Ubuntu server in directory /usr/local/. A path to the Linaro tool chain is defined in the script studentStartScript.sh.

### Build the Executable

- To use the cross compiler to build the executable, cd the terminal to the directory where example1.c was stored and use the following command: 

arm-linux-gnueabihf-gcc -o example1 –g example1.c

The cross compiler tools will compile the file and build an executable called example1 in the same directory.

- To verify that the compilation was done for the ARM processor and not for the native Intel (or other) processors do the following:

sudo file example1

- The results should show ARM architecture: 

“example1: ELF 32-bit LSB executable, ARM, version 1 (SYSV), dynamically linked (uses shared libs), for GNU/Linux 2.6.31, BuildID[sha1]=0x953dac672e7159d481d5a6d3bbb5356e5f870d21, not stripped”

### Unzip and Decompress the File System and add the new executable

The compressed file system has a cpio.gz extension (the current release name for the file system is tisdk-rootfs.cpio.gz)

- If it does not already exist, create a new directory /opt/filesys/studentN

sudo mkdir /opt/filesys/studentN

cd /opt/filesys/studentN

- Copy the current compressed file system to the new directory:

sudo cp /var/lib/tftpboot/studentN/tisdk-rootfs.cpio.gz

- Unzip the compressed file system

sudo gzip –d tisdk-rootfs.cpio.gz   (or name of the file system that you choose)

- Uncompress  the file system from the cpio file. This operation builds the complete file system. NOTE: The compressed file tisdk-rootfs.cpio is still in the directory  

sudo cpio –i –F tisdk-rootfs.cpio

- Remove tisdk-rootfs.cpio

sudo rm tisdk-rootfs.cpio

- Copy the executable that was built previously (example1) into the usr/bin  directory in the file system. The complete path is /opt/filesys/studentN/usr/bin

sudo cp <example1DirectoryLocation>/example1  /opt/filesys/studentN/usr/bin/.

### Compress and Zip the New File System

- The next step is to compress the file system back into a new file system. This is done by piping all the directories and the files in the file system into the cpio. The resulted compressed file system will be copied to one directory above:

cd /opt/filesys/studentN

sudo chmod -R 777 *     (Provides read/write/execution permission
                                                     to all files and subdirectories)

sudo find .  | sudo cpio –H newc –o –O  <new.cpio>

Where <new.cpio> is the new compressed file system

- To zip the new file system, do the following:

sudo gzip new.cpio        (this will generate a file new.cpio.gz)

sudo cp new.cpio.gz   /var/lib/tftpboot/studentN/.

- At this point, studentN has two file systems. There are several options for making the new one active:

  - The user can change the environment variable  name_fs in the EVM U-BOOT to new.cpio.gz

  - The user can delete the previous tisdk-rootfs.cpio.gz, and then change the name of new.cpio.gz to tisdk-rootfs.cpio.gz

  - The user can change the name of new.cpio.gz to any other name (with the .cpio.gz extension) like tisdk-rootfs-example1.cpio.gz, and change the environment variable name_fs in the EVM U-BOOT to the new name.

## Reboot the EVM and Run the New Program

- Reboot the EVM with the new file system.

- After boot, login as a root.

- Go to /usr/bin and run example1.

- Observe the results.

### Task 2 (optional): Build U-boot, Boot Monitor and Kernel

Linux distribution enables users to download all source code and build the Linux components. This optional task gives instructions how to do so.

TI uses the Arago distribution on a public server to store all source code. The user can download all the sources and make files to build all images in the release. The Ubuntu server must be connected to the network to facilitate downloading of the source code. If the server is behind a firewall, proxies must be set. These proxies are defined already in the /usr/local/studentStartScript.sh script.

### U-BOOT Source Code Extraction

Before getting the source, the user must be in his home directory. Cloning the source code will generate sub-directories.

- cd  ~/     back into the home directory /home/student

- git clone git://arago-project.org/git/projects/u-boot-keystone.git

This instruction will clone the repository into your local directory ~/u-boot-keystone

- cd u-boot-keystone    Change directory to where the source is located

- git reset --hard <label>   Where label is the label that is attached to the current release. To find the head label, the user should look at the release notes. For MCSDK 3_14 the label is K2_UBOOT_2013-01_13.09_01. So the reset instruction for the current release will be the following:

git reset –hard K2_UBOOT_2013-01_13.09_01

### U-BOOT Build Instructions

Different versions of the U-BOOT can be built, depending on how the code is loaded. U-BOOT can be loaded from Code Composer Studio or from boot methods. The format can either be u-boot.bin, u-boot.img, u-boot-spl.bin or u-boot-spi.gph

- Option A: if using CCS to load (u-boot.bin)

make tci6638_evm_config

make

- Option B: if using the two stage SPI NOR boot

make tci6638_evm_config

make spl/u-boot-spl.bin

make tci6638_evm_config

make u-boot.img

make tci6638_evm_config

make u-boot-spi.gph

### Boot Monitor (skern.bin) Source Code Extraction and Build Instructions

Before getting the source, the user must be in his home directory. Cloning the source code will generate sub-directories.

- cd  ~/     back into the home directory /home/student

- git clone git://arago-project.org/git/projects/ boot-monitor.git

This instruction will clone the repository into your local directory ~/boot-monitor

- cd  boot-monitor change directory to where the source is located

- git reset --hard <label>   Where label is the label that is attached to the current release. To find the head label, the user should look at the release notes. For MCSDK 3_14 the label is K2_BM_13.08. So the reset instruction for the current release will be the following:

git reset –hard K2_BM_13.08

- Build the boot monitor:
 make clean
 make

### Linux Kernel and Device Tree  Source Code Extraction and Build Instructions

Before getting the source, the user must be in his home directory. Cloning the source code will generate sub-directories.

- cd  ~/     back into the home directory /home/student

- git clone git://arago-project.org/git/projects/ linux-keystone.git

This instruction will clone the repository into your local directory ~/boot-monitor

- cd  linux-keystone  Change directory to where the source is located

- git reset --hard <label>   Where label is the label that is attached to the current release. To find the head label, the user should look at the release notes. For MCSDK 3_14, the label is K2_LINUX_03.08.04_13.09_01. So the reset instruction for the current release will be the following:

git reset –hard K2_LINUX_03.08.04_13.09_01

- Build the files:

make keystone2_defconfig

make uImage

make keystone-sim.dtb

make tci6638-evm.dtb

The built files:

vmlinux is in /linux-keystone folder:

uImage & *.dtb are in /linux-keystone/arch/arm/boot folder

# Lab 3: Boot Using NFS-mounted File System

## Purpose

The purpose of this lab is to demonstrate how to boot the EVM when the file system resides on a different server that is mounted on the EVM, then develop code on the Linux host and move it to the file system. The executable will be available to the ARM on the EVM.  A debug session using gdb will be performed from the serial port terminal.

### Task 1: Build a file system on a Linux Host, Use the NFS server

The NFS server is installed on the Ubuntu server in the directory /opt/filesys. Each student has a sub-directory where he or she builds the file server, and the Uboot is configured to reach this directory for each student. The file system to be mounted should be built on the local Ubuntu machine.

- Create a directory where the file system resides; say /opt/filesys/studentN (where N is the student number). NOTE: This directory should already exist.

- Copy a tar version of the compressed file system tisdk-rootfs.tar.gz (part of the release in the images directory) into /opt/filesys/student

- Untar the file system:  sudo tar zxf tisdk-rootfs.tar.gz

- Delete the original compress file: sudo rm tisdk-rootfs.tar.gz

- Add the file system directory to the exports list. Open the file /etc/exports and add the following line to it (or simply verify that it is already there):

/opt/filesys *(rw,subtree_check,no_root_squash,no_all_squash,sync)

The file /etc/exports should appear as follows:

- The instructor will start the NFS server

### Task 2: Configure U-BOOT to mount the file server and boot

- Power cycle the EVM. In the ARM tera-terminal window, stop the autoboot.

- Change the following environment variables:

  - Change the boot to be from the network :

setenv boot net

  - Add the nfs server ip

setenv nfs_serverip  xxx.xxx.xxx.xxx   

Where xxx.xxx.xxx.xxx is the IP address of the Ubuntu server on which the file system resides. For the standard workshop, the address is 192.168.0.100

  - Define the file system root directory:

setenv nfs_root  /opt/filesys/student

  - Configure the arguments for the boot: 

setenv args_net 'setenv bootargs ${bootargs} rootfstype=nfs root=/dev/nfs rw nfsroot=${nfs_serverip}:${nfs_root},${nfs_options} ip=dhcp'

  - Save the new environment variables:

saveenv

- Boot the EVM.

NOTE: If the DHCP does not supply an IP address to the EVM, the EVM will use its default IP address. This default IP address is defined in the environment variables as ipaddr. If this does not exist, the user can configure it using setenv ipaddr yyy.yyy.yyy.yyy

### Task 3: Build a new C program in the file system, and debug it

- In a local viewer to the Ubuntu, go to /opt/filesys/studentN and look at the file system.

- Follow the Example Simple Code section of the previous lab to copy example1.c into one of the directories of the file system (For example, /opt/filesys/studentN/bin).

NOTE: You may need to modify permissions of the bin directory as follows:

sudo chmod 777 /opt/filesys/studentN/bin

- Set the terminal in the bin directory: cd /opt/filesys/student/bin

- Compile and build the application similar to the method used in the previous lab, but add the debug flag (-g) to the command:

arm-linux-gnueabi-gcc –g  -o example1 example1.c

- Back to the tera-terminal, navigate to the /bin directory: cd /bin

  - Make sure that example1.c  and example1 are both  in the bin directory

ls –ltr example1*

  - Start a debug session:

gdb example1

  - Simple gdb commands:

    - list		see the source

    - b 		set a break point

    - r 		run to the break point

    - s 		step

    - n 		next (step over)

    - c 		run to the next breakpoint

    - finish	end

  - There are many gdb quick guides on the Web.  Here is a URL to one of them:

http://condor.depaul.edu/glancast/373class/docs/gdb.html

# Lab 4: Build, Run and Optimize DSP Project using CCS

## Purpose

In previous labs, you developed and debugged an ARM program.  The purpose of this lab is to develop and debug a multicore C66x program using CCS IDE. This lab has the following parts:

- Using CCS, build a simple FIR project that runs on a single core.

- Optimize the code by achieving software pipeline, understand what can prevent the compiler from generating software pipeline code.

- Optimize execution by enabling cache.

- Perform parallel processing of the code and observe multi-cores processing speedup.

CCS IDE is used to execute the lab.

Before starting, the EVM should be configured to no-boot mode. To do so, set the dipswitch (SW1) on the EVM to: 1 Off   2 Off    3 Off    4On

## Project Files

The following files are used in this lab:

- firMain.c

- firRoutines.c.c

- firTest.cmd

- firTest.h

- utilities.c

- Utilities.h

- firFilterLab.cfg

### Task 1: Build and Run the Project

- FTP into the Ubuntu server and get all the files in directory /usr/local/projects/DSP/firLab  into the local directory c:\ti\labs\firFilter on your PC. If this directory does not exist, create it.

- Open CCS.

- Create new project through the CCS menu item File  New  CCS Project.

- Enter firLab1 as a Project Name.

- Click the check box to Use default location.

- Set the Family to C6000 and Variant to Generic C66xxx Device

- Then press Finish to create the new project.  See the screen shot.
NOTE: You will use the default location and not the location in the screen shot.

- Then in the Project Explorer view, right-click on the newly-created firLab1 project, and click on Add Files…

- Browse to C:\ti\labs\firFilter, select all the files in this directory, and click Open. When prompted how files should be imported into the project, leave it as default of Copy File. If you defined the new project with main.c, remove the main.c file that may be created.

- As soon as the file firFilter.cfg is imported into the project, CCS will ask you to enable RTSC support. See the screen shot below. Select Yes.

- Open Project Properties and select general->RTSC. Look at the RTSC modules that are selected in the screen shot below and make sure that you select ONLY the same RTSC modules (or packages). When a project starts, RTSC attempts to include all the modules in the release. So unselect any module that is not in the screen shot.

NOTE: The TARGET CONTENT BASE should reflect the location of CCS on your system.

- Click on the platform tab and select ti.platform.evmTCI6638K2K  platform as shown in the next screen shot.

NOTE: RTSC projects require the user to select three types of information.

- The device family in the CCS create page determines what core is used and thus what version of the compiler should be used (different cores have different intrinsic functions).

- The platform that is defined here determines the memory configuration of the core.

- To build the correct RTSC drivers, the device name should be defined. This is done by adding a predefined symbol with the device name. More about it later.

- If you try to build the project now, you will receive a long list of errors such as the following: 

C:/ti/MCSDK_3_14/pdk_keystone2_3_00_02_14/packages/ti/csl/csl_cacheAux.h", line 86: error #20: identifier "CSL_C66X_COREPAC_REG_BASE_ADDRESS_REGS" is undefined

This is because the device being used is not yet defined. So RTSC does not know how to build the drivers for the device that is used. The next step describes how to configure the device

- Currently, the file cslr_device.h refers to two devices, K2K and K2H. The EVM uses the K2K device:

#if defined(DEVICE_K2K)

#include <ti/csl/device/k2k/src/cslr_device.h>

#elif defined(DEVICE_K2H)

#include <ti/csl/device/k2h/src/cslr_device.h>

#endif /* DEVICE_XXXXX */)

- To configure the device, open the project properties, build -> C6000 compiler ->advanced  Options -> predefined symbols and enter DEVICE_K2K as the next screen shot shows.

- Click OK and then OK again to go to the properties page.

- Right click on the project name and select rebuild.  If the build goes correctly, you will see the following in the console window.

NOTE: Look at the debug directory to ensure that the file firLab1.out is there. Ignore any warnings.

- Examine the code in firMain.c. There are five cases, but only case 1 is not commented out.

  - DSP 0 generates input data (inputData) and a set of filter coefficients (filterCoef)

  - Depending on the case, a set of fir filters is applied to the data and the results are written to the out file (outputFilter).

  - A set of timer registers (TSCL and TSCH) measure the execution time of the fir filter.

  - The standard printf function prints the results on the console.

### Task 2: Define the Target

In this lab, the DSP code is run from no-boot mode. The no-boot mode requires setting the dipswitch SW1 of the EVM to: 1 Off    2 Off   3 Off   4 On.

Since no-boot mode is chosen, the device configuration (DDR configuration, PLL configuration and so on) must be done in a gel file.

#### Create a new target in CCS

- Create a new target configuration:

  - Select the CCS menu option View  Target Configurations.

  - Select User Defined.

  - Right-click and select New Target Configuration.

- Enter the name of the new target configuration in the File Name: text box.

  - Set the File name based on the EVM model, <model>.ccxml
For example, ‘TCI6638.ccxml’

  - Leave the Location the default value:
“C:\Documents and Settings\student\ti\CCSTargetConfigurations”

  - Click the Finish button. The .ccxml file will now open in a GUI-based view with the Basic tab active.

- Define the new target configuration by selecting the connection type in the Basic Tab.

- The Connection drop-down menu identifies the emulator type. For example, ‘Texas Instruments XDS2xx USB Emulator.”

  - Board or Device identifies the TI processor device, set it to 6638 and select TCI6638

  - Under Save Configuration, click the Save button.

- Configure setup in Advance Tab

  - Click the Advanced tab at the bottom of the screen.

  - Select Core 0 on the target device:

    - TCI6638_0  IcePick_C_0  Subpath_1  C66xx_0

  - You will now see a sub-window called Cpu Properties that allows you to choose an initialization script.

  - Locate the appropriate GEL file, then click Open:

    - Select: C:\ti\CCSv5_4\ccsv5\ccs_base\emulation\boards\evmtci6638k2k\gel\evmtci6638k2k.gel

    - Repeat the process for all C66x cores, that is C66xx_1, C66xx_2, … C66xx_7

Click the Save button.

### Task 3: Connect to the EVM

- Click the Open Perspective (available at the right top corner of the CCS window).

- Switch to the Debug Perspective by selecting the CCS menu option Window  Open Perspective  CCS Debug.

- Select the CCS menu option View  Target Configurations. Select the target configuration you created

- Launch the target configuration as follows:

  - Select the target configuration .ccxml file.

  - Right click and select Launch Selected Configuration.

- This will bring up the Debug window. NOTE: This may take some time, but you will eventually see all the device cores.

  - Select all C66x cores (select + Ctrl)

  - Right click and choose group cores.

  - Select the group, then right click and select Connect Target.

### Task 4: Load and Run CASE 1

- Select the core group and load the .out file created earlier in the lab.

  - Select the CCS menu option Run  Load  Load Program

  - Click Browse project…

  - Select firLab1.out by unwrapping the firLab1->Debug and click OK.

  - Click OK to load the application to the target (all Cores).

- Run the application by selecting the CCS menu option Run  Resume.

- A successful run should produce a console output as shown below. Record the cycles time:

[C66xx_0]  start generating input data

finish generating input data

case 1 ->  time consumed By core -> 0 610749952.000000

QUESTION:

Look at the function  CACHE_disableCaching ((Uint8) 144) which disables cache-ability for memory region. What memory region is it?

See Table 4-20 in the C66 CorePac User’s Guide http://www.ti.com/lit/ug/sprugw0c/sprugw0c.pdf

Look at the User Guide, the code, and the map file, and fill out the table below.

| Command | Memory region | Variables in this region |
| --- | --- | --- |
| CACHE_disableCaching ((Uint8) 128) |  |  |
| CACHE_disableCaching ((Uint8) 136) |  |  |
| CACHE_disableCaching ((Uint8) 144 |  |  |
| CACHE_disableCaching ((Uint8) 145) |  |  |
| CACHE_disableCaching ((Uint8) 146) |  |  |
| CACHE_disableCaching ((Uint8) 147) |  |  |

### Task 5: Use Optimization and Disable Symbol Debug for the fir Filter Routine

As the project is still in development/debug state, there is no optimization and full debug support.  The next step is to optimize the fir filter and disable the debug information.  However, leave the other parts of the project without optimization and with full debug support. The properties for the file firRoutines.c will be changed. No other file will be effected.

- In the project explorer, select the file firRoutines.c and right click. Open the properties dialogue window as shown below.

- Select  build->optimization. In the dialogue window set optimization to 3

- From the debug options dialogue select suppress all symbolic debug generation from the pull-down menu.

- From the build ->C6000 compiler -> Advanced Debug, select Assembly options and check Keep the generated assembly language (.asm) file as shown in the screen shot below.

- Click OK and rebuild the project. Load and run.

- A successful run should produce a console output as shown below. Record the cycles time:

Start generating input data

Finish generating input data

Case 1: Time consumed by core -> 0 500579008.000000

QUESTION:

Is the code really optimized?  Only 15% improvement.

Look at the assembly file firRoutines.asm in the debug directory and search for the function firRealFilter.  Look for the loop and see if the compiler could get software pipeline

What is the reason that the loop does not qualify for software pipeline?

### Task 6: Optimize Software Pipeline

The reason why the fir filter loop is not qualified for a software pipeline is because it calls myMultiply. The next task is to inline this function. myMultiply is an artificial function (i.e., no one will develop this function in real code). So it is easy to “inline” it. Look at the definition of myMultiply in the utilities.c file and inline it.

- Change the function firRealFilter by inline myMultiply function

- Save and build the project. Load and run.

- A successful run should produce a cosole output as shown below. Record the cycle time:

start generating input data

finish generating input data

case 1 ->  time consumed By core -> 0 273086080.000000

- Next, tell the compiler the minimum number times that each loop will be executed. The filter size in this program is 8. Assume that the filter size will always be more than 4 and divided by 4, so adding a pragma(  #pragma MUST_ITERATE(4,,4); ) will tell the compiler that the inner loop must be performed at least 4 times and the number of iterations is divided by 4.

- The outer loop presents the size of the output vector. The number of elements is 16K, but eventually we would like to run it on all 8 cores, so each core will have about 2K element.  It is enough if we tell the compiler that the number of elements is more than, say 64. However, if you look carefully, you will notice that the number of output results is 16K – filter size + 1, so this is an odd number. You can tell the compiler that the number of elements is more than 64. In that case use something like (#pragma MUST_ITERATE(64,,1); ) or, if you agree to ignore the last fake result, you can tell the compiler (#pragma MUST_ITERATE(4,,2); ))

- Add the two pragma directives before the two loops (internal and external) in the function save and build.

- If the external loop is pragma MUST_ITERATE(64,,1)

start generating input data

finish generating input data

case 1 ->  time consumed By core -> 0 221407008.000000

- If the external loop is pragma MUST_ITERATE(64,,2);

start generating input data

finish generating input data

case 1 ->  time consumed By core -> 0 221306848.000000

QUESTION: To summarize the code optimization section, complete the following table:

| Optimization Technique | Cycles | Improvements compared with previous line |
| --- | --- | --- |
| No Optimization |  |  |
| Compiler optimization 3, no symbolic debug |  |  |
| Software Pipeline |  |  |
| Adding pragma must iterate |  |  |

### Task 7: Enable the Cache

Enabling the cache is done in CASE 2.  Un-comment the line #define CASE_2 above the main() in firMain.c

QUESTION:

What instruction(s) enable the cache?

The function    CACHE_enableCaching ((Uint8) 128)    ; was discussed in Task 4. The function    CACHE_setL2Size ((CACHE_L2Size) 4); is part of the file csl_cachAux.h in the \MCSDK_3_14\pdk_keystone2_3_00_02_14\packages\ti\csl directory. Note, version number and location of MCSDK may be different for your setting.

- Un-comment the line #define CASE_2 in firMain.c

- Save, build, load and run.  The results will look like the following:

start generating input data

finish generating input data

case 1 ->  time consumed By core -> 0 221223008.000000

case 2 ->  time consumed By core -> 0 7491409.000000

QUESTION: Complete the table.

| Optimization Technique | Cycles | Improvements compare with previous line |
| --- | --- | --- |
| No Optimization |  |  |
| Compiler optimization 3, no symbolic debug |  |  |
| Software Pipeline |  |  |
| Adding pragma must iterate |  |  |
| Enabling cache |  |  |

QUESTION: What are the most important steps to optimize code running on a single core?

### Task 8: Running in Parallel on Multiple Cores

Multiple cores are enabled in CASE 3 (2 cores), CASE 4 (4 cores) and CASE 5 (8 cores).

Un-comment the lines #define CASE_3 #define, CASE_4  and  #define CASE_5 above the main() in firMain.c

- Un-comment the line #define CASE_3 #define CASE_4  #define CASE_5   in firMain.c

- Save, build, load and run.  The results will be look like the following:

finish generating input data

case 1 ->  time consumed By core -> 0 288423616.000000

case 2 ->  time consumed By core -> 0 7493824.000000

case 3 ->  time consumed By core -> 0 3680093.000000

[C66xx_1] case 3 ->  time consumed By core -> 1 3678251.000000

[C66xx_0] case 4 ->  time consumed By core -> 0 1839643.000000

[C66xx_1] case 4 ->  time consumed By core -> 1 1838608.000000

[C66xx_2] case 4 ->  time consumed By core -> 2 1839438.000000

[C66xx_3] case 4 ->  time consumed By core -> 3 1836440.000000

[C66xx_0] case 5 ->  time consumed By core -> 0 918711.000000

[C66xx_1] case 5 ->  time consumed By core -> 1 921884.000000

[C66xx_2] case 5 ->  time consumed By core -> 2 921973.000000

[C66xx_3] case 5 ->  time consumed By core -> 3 920785.000000

[C66xx_6] case 5 ->  time consumed By core -> 6 922374.000000

[C66xx_4] case 5 ->  time consumed By core -> 4 923078.000000

[C66xx_5] case 5 ->  time consumed By core -> 5 921646.000000

[C66xx_7] case 5 ->  time consumed By core -> 7 920075.000000

For each case, the total time that is consumed to perform the FIR filter is the maximum time of all the cores.

QUESTION: Complete the table

| Case | Cycles per core | Execution cycles (this is the cycles of the core with the highest cycles count) | Accumulate execution time for all the cores | Penalty of the accumulation execution time compared to single core (CASE 2) |
| --- | --- | --- | --- | --- |
| Case 2 – single core |  |  |  |  |
| Case 3 – 2 cores |  |  |  |  |
| Case 4 – 4 cores |  |  |  |  |
| Case 5 – 8 cores |  |  |  |  |

QUESTIONS:

- What is the purpose of the function waitBarrier(barrier_1, coreNum, jointNumber)? What would happen if the function is commented out?

Try to comment out the function (3 places) and see what happens.

- What is the purpose of the function waitAboutNSeconds(10) inside the function  waitBarrie? What would happen if the function is commented out? Do you understand why?

Try to comment out the function and see what happens.  Think about timing between cores.

- Can you think about other (better) methods to synchronize the execution of all the cores?

Semaphores?, QMSS queues based solution?, openMP? .

# Lab 5: Load and Run DSP Code Using MPM Server

In this lab, you build a DSP project similar to the previous lab. Before starting, you should change the boot mode of the EVM to no-boot mode. Set SW1 of the EVM back to 1 Off  2 Off  3 On  4 Off.

Read the instructions in Lab 3 to ensure that the EVM boots using NFS-mounted file system.

## Purpose

Building a DSP code that is managed by the ARM. The ARM will reset C66x Core 0, load it with an executable, run it, and retrieve the results.

## Project Files

The following files are used in this lab:

- Main.c

- mpmsrv_keystone2_example1.cfg

### Task 1: Build and Run the Project

- ftp into the Ubuntu server and get all the files that are in directory /usr/local/projects/DSP/mpm  into a local directory c:\ti\labs\mpm_example on your PC.  If this directory does not exist, create it.

- Open CCS.

- Create new project through the CCS menu item File  New  CCS Project.

- Enter mpm_example as a Project Name.

- Click the check box to Use default location.

- Set the Family to C6000 and Variant to Generic C66xxx Device

- Then press Finish to create the new project

- Then in the Project Explorer view, right-click on the newly-created mpm_example project, and click on Add Files…

- Browse to C:\ti\labs\mpm_example, select all the files in this directory, and click Open. When prompted how files should be imported into the project, leave it as default of Copy File. If you defined the new project with main.c, remove the main.c file that may be created.

- As soon as the file mpmsrv_keystone2_example1.cfg imported into the project, CCS will ask you to enable RTSC support, Select yes.

- Open Project Properties and select general->RTSC. Look at the RTSC modules that are selected in the screen shot below and make sure that you select ONLY the same RTSC modules (or packages). When a project starts, RTSC attempts to include all the modules in the release. So unselect any module that is not in the screen shot.

NOTE: The TARGET CONTENT BASE should reflect the location of CCS on your system.

- Click on the platform tab and select ti.platform.evmTCI6638K2K  platform

NOTE: RTSC projects require the user to select three types of information.

- The device family in the CCS create page determines what core is used and thus what version of the compiler should be used (different cores have different intrinsic functions).

- The platform that is defined here determines the memory configuration of the core.

- To build the correct RTSC drivers, the device name should be defined. This is done by adding a predefined symbol with the device name. More about it later.

- Right click on the project name and select rebuild.  If the build goes correctly, you will see the following in the console window. NOTE: Look at the debug directory to ensure that the file MPM_example.out is there. Ignore any warnings.

'Building target: MPM_example.out'

'Invoking: C6000 Linker'

"C:/ti/CCSv5_4/ccsv5/tools/compiler/c6000_7.4.2/bin/cl6x" -mv6600 --abi=eabi -g --display_error_number --diag_warning=225 --diag_wrap=off -z -m"MPM_example.map" -i"C:/ti/CCSv5_4/ccsv5/tools/compiler/c6000_7.4.2/lib" -i"C:/ti/CCSv5_4/ccsv5/tools/compiler/c6000_7.4.2/include" --reread_libs --warn_sections --display_error_number --diag_wrap=off --xml_link_info="MPM_example_linkInfo.xml" --rom_model -o "MPM_example.out" -l"./configPkg/linker.cmd"  "./main.obj" -l"libc.a"

<Linking>

'Finished building target: MPM_example.out'

' '

**** Build Finished ****

### Task 2: Using MPM to load, run and observe results

In this part, we assume that the EVM is boot in net mode, that is, the file system is on the server and it is mounted to the EVM as used in Lab3

- FTP the out file into the server to /opt/filesys/studentN/bin where N is the student number.

- Reboot the EVM using NFS.

- From the terminal login as root

- cd /bin

- Use MPM to reset, load, and run core 0 with MPM_example.out  by using the following MPM commands:

mpmcl reset dsp0

mpmcl load dsp0 MPM_example.out

mpmcl run dsp0

- After the end of run look at the trace buffer printing by using the following command

cat  /debug/remoteproc/remoteproc0/trace0

- Change the main.c file as you wish, build it again, ftp to the file system (step 7) load the code to a different dsp (use N here) and run it:

mpmcl reset dspN

mpmcl load dspN MPM_example.out

mpmcl run dspN

- After the end of run look at the trace buffer printing by using the following command:

cat  /debug/remoteproc/remoteprocN/trace0

# Lab 6: ARM-DSP Communication Using MPM & Shared DDR

In this lab, you build a DSP project similar to Lab 4 that uses the DDR. Unlike the previous Lab where the code and the data were only in L2, in this lab some DDR is used by the DSP.

## Purpose

Building a DSP code that uses the DDR and is managed by the ARM.

## Linux and DSP simple memory management

The previous project uses private L2 memory for program and data. This DSP project uses DDR.  How does the system manage the DDR resources between the DSP and the ARM?

The Linux uses part of the DD. So if a DSP program uses some of the DDR, it must tell the Linux.  This is done in the U-BOOT environment.

To do it correctly, the user must follow the following steps:

- Stop autoboot and look at the messages from the U-BOOT. It looks like the following:

I2C:   ready

Detected SO-DIMM []

DRAM:  1 GiB

NAND:  512 MiB

Net:   TCI6638_EMAC, TCI6638_EMAC1

Hit any key to stop autoboot:  0

The size of the DRAM is 1 GiB in this case. It can be 2G or 8 G depends on the EVM revision

- Now determine how much DRAM the DSP needs. Obviously it must be less than the total DRAM in the EVM. Assume that Linux uses 512M and the DSP will use 512M. The user must tell the U-BOOT that 512M is reserved for the DSP.

- After stopping the autoboot, configure the memory that is assigned to the DSP (if it is not configured already).

setenv   mem_reserve   512M   (For EVM with 2G DRAM it can be 1536M
                                                                         or even larger if the DRAM is more than 2G)

saveenv   and boot

- The memory that is reserved for the DSP is located at the end of the available memory. For the 1G DRAM case, available memory is between 0x80000000 and 0xbfffffff, so the 512M reserved for the DSP start at address 0xa0000000 to address 0xbfffffff

- Next, you need to build the DSP code and ensure that it uses only the assigned DDR.  One way to do this when using RTSC is to re-define the platform.  The release platform defines DDR starting at 0x80000000 and with size 0x80000000.

- To modify the platform in CCS, go to the debug perspective tools -> RTSC Tools ->platform -> edit/view. Make sure that the repository is the location of XDCTools/packages (in MCSDK_3_14 setting it is in “MCSDK_3_14\xdctools_3_25_02_70\packages” and click on package name. Choose the package that corresponds to the EVM, (the current EVM is  ti.platforms.evmtci6638K2K), CPU core is CPU, and click NEXT.

Changing the platform as illustrated in the following screen shot.

- In a link command file XXX.cmd, make sure that all DDR sections are assigned to the memory DDR3_ForDSP (in the case above). Verify the correct address in the map file after rebuilding the project.

- As was mentioned before, the DSP in this lab does not use the DDR. So for this lab, the above procedure is not necessary. However, it can be used as reference for more complex projects.

## Project Files

The following files are used in this lab:

- Main.c

- mpmsrv_keystone2_example1.cfg

### Task 1: Build and Run the Project

- FTP into the Ubuntu server and get all the files that are in directory /usr/local/projects/DSP/mpm  into a local directory on your Laptop c;\ti\labs\mpm_example.  If this directory does not exist, create it

- Open CCS.

- Create new project through the CCS menu item File  New  CCS Project.

- Enter mpm_example as a Project Name.

- Click the check box to Use default location.

- Set the Family to C6000 and Variant to Generic C66xxx Device

- Then press Finish to create the new project

- Then in the Project Explorer view, right-click on the newly-created mpm_example project, and click on Add Files…

- Browse to ‘C:\ti\labs\mpm_example,’ select all the files in this directory, and click Open. When prompted how files should be imported into the project, leave it as default of Copy File. If you defined the new project with main.c remove the main.c file that may be created.

- As soon as the file mpmsrv_keystone2_example1.cfg imported into the project, CCS will ask you to enable RTSC support, Select yes.

- Open Project Properties and select general->RTSC. Look at the RTSC modules that are selected in the screen shot below and make sure that you select ONLY the same RTSC modules (or packages). When a project starts, RTSC attempts to include all the modules in the release. So unselect any module that is not in the screen shot.

NOTE: The TARGET CONTENT BASE should reflect the location of CCS on your system.

- Click on the platform tab and select ti.platform.evmTCI6638K2K  platform

NOTE: RTSC projects require the user to select three types of information.

- The device family in the CCS create page determines what core is used and thus what version of the compiler should be used (different cores have different intrinsic functions).

- The platform that is defined here determines the memory configuration of the core.

- To build the correct RTSC drivers, the device name should be defined. This is done by adding a predefined symbol with the device name. More about it later.

- Right click on the project name and select rebuild.  If the build goes correctly, you will see the following in the console window.

NOTE: Look at the debug directory to ensure that the file firLab1.out is there. Ignore any warnings.

'Building target: MPM_example.out'

'Invoking: C6000 Linker'

"C:/ti/CCSv5_4/ccsv5/tools/compiler/c6000_7.4.2/bin/cl6x" -mv6600 --abi=eabi -g --display_error_number --diag_warning=225 --diag_wrap=off -z -m"MPM_example.map" -i"C:/ti/CCSv5_4/ccsv5/tools/compiler/c6000_7.4.2/lib" -i"C:/ti/CCSv5_4/ccsv5/tools/compiler/c6000_7.4.2/include" --reread_libs --warn_sections --display_error_number --diag_wrap=off --xml_link_info="MPM_example_linkInfo.xml" --rom_model -o "MPM_example.out" -l"./configPkg/linker.cmd"  "./main.obj" -l"libc.a"

<Linking>

'Finished building target: MPM_example.out'

' '

**** Build Finished ****

# Lab 7: Build and Run ARM Code Using CCS on Linux

## Purpose

The goal of this lab is to demonstrate some basic operations using CCS on ARM code as preparation for EVM debug from CCS.

## Prerequisites

CCS must be installed on the Linux machine.

## Task 1: CCS - Start a new cross compiler project and build it

- To start CCS, go to CCS eclipse directory and enter the following:

sudo ./ccstudio

- After CCS start, and in the edit perspective, start a new (not CCS) project:

File -> New -> Project  (Not CCS Project)

C/C++  and then C project

- From the C project, choose Hello World ANCI C project, Toolchains Linux GCC, give the project a name (myHello), and finish.

- Before continuing, to make the Lab a little more interesting, you can change the source code using CCS editor.  The following is a version of myHello.

int main(void)

{

int  i,  j, k, l    ;

printf("printf command start the program   \n")  ;

puts (" puts command start the program  \n")  ;

l = 12   ;

for (i=0; i<1000; i++)

{

j = l + i   ;

k = 2 * j   ;

l =  k + j  ;

if (l > 10000)

{

printf(" inside loop, l > 10000  i= %d k = %d j = %d l = %d  \n", i,k,j,l) ;

l = l - 10000    ;

printf(" after  the normalization  i= %d k = %d j = %d l = %d  \n", i,k,j,l) ;

}

}

printf(" END ->>>  i= %d k = %d j = %d l = %d  \n", i,k,j,l) ;

puts("!!!Hello World!!!"); /* prints !!!Hello World!!! */

return 1;

}

- Next configure the project properties. Right click on the project and start properties. Open C/C++ build and open settings. In the dialogue window select GCC C compiler and enter arm-linux-gnueabihf-gcc. The same tool will be set for the linker as well.

For assembly, choose the arm-linux-gnueabihf-as assembler.

Next the include path to the tools should be configured. From project properties select

C/C++ General->Path and Symbols

Note: Depending on project requirements, more paths may be needed

	 From Includes tab Add GNU C Includes. The easier way is by going to ADD and then file system and browse:
	DirectoryWhereTheToolsAre/gcc-linaro-arm-linux-gnueabihf-4.7-2013.03-20130313_linux/arm-linux-gnueabihf/libc/usr/include
	DirectoryWhereTheToolsAre /gcc-linaro-arm-linux-gnueabihf-4.7-2013.03-20130313_linux/arm-linux-gnueabihf/libc/usr/include/arm-linux-gnueabihf
	- Add to all configurations

From Library Paths tab Add Library Paths

DirectoryWhereTheToolsAre /gcc-linaro-arm-linux-gnueabihf-4.7-2013.03-20130313_linux/arm-linux-gnueabihf/libc/usr/lib/arm-linux-gnueabihf
	- Add to all configurations

Last the path to the tools should be configured. From project properties select

- Right-click Project, select Properties

- Select C/C++ BuildEnvironment

- Click Add…

Enter Name: PATH, Add to all configurations

Press OK  (ccs will automatically populate with internal path)

- Select PATH, click Edit…

- Prefix string with path to toolchain binaries
     (e.g. /opt/gcc-linaro-arm-linux-gnueabihf-4.7-2013.03-20130313_linux/bin)

- Path must be delimited with a colon ‘:’

Do clean build and build

The consol should look like the following:

## Task 2: GDB from the tera-terminal

There are multiple ways to use gdb to run and debug myHello. It can be done outside of CCS or within CCS.

### Move the Debug directory outside of CCS

- First allocate where the program is located in the linux host system. From property -> resources you will see something like:

/root/workspace_v5_4/myHello

Where workspace_v5_4 is the default workspace for CCS.

- An alternative method is to search for the myHello project:

cd /     takes us to the start of file system

sudo find . –name myHello –print       (look for all files with the name myHello in them)

- Go to the myHello directory and change the permissions on the Debug sub-directory

sudo chmod 777 Debug

- move the Debug directory to the EVM  file system (if the default setting was used, it is in /opt/filesys)

cd /opt/filesys

cd bin

sudo cp –R /root/workspace_v5_4/myHello/Debug  .

- From the tera-terminal go to the bin subdirectory and look at the Debug subdirectory. The src sun-directory should have the source code.

- Start the gdb from the tera-terminal

gdb myHello

display the source code

list

- Put a breakpoint after the printf

b 17

- run the code and observe the printing on the terminal

r

c

You can play with other gdb instructions.

## Task 3: Connect a SSH terminal into the target

- After reboot and login the target from the tera-terminal,  do ifconfig eth0 and see what the IP address of the target is. Note, the IP address is assigned by the DHCP server.

- Start CCS; Select WindowOpen PerspectiveOther
Open Perspective: Check Show All, Select Remote System Explorer
Allow enablement of “Remote System Explorer Tools”

- Select CCS Edit perspective

- Select WindowShow ViewOther
Expand Remote Systems, Select Remote Systems
Optionally drag Remote Systems to different location
(next to Project Explorer tab is a nice spot…)

- From Remote Systems window, Right-ClickNewConnection… in the window, select linux and click next

- Configure the remote linux connection, enter the IP address as the host name, give connection name and profile name:

- Files, select ssh.files

- Click Next

- Select processes.shell.linux

- Click Next

- Select ssh.shells

- Click Next

- Select ssh.terminals

- Finish

- Opening a terminal:
        Right-Click, Select Launch Terminal
        Enter password (for root user name leave the password blank)

After warnings a terminal will be opened into the target ARM

Note – the green arrow indicates that the connection is good. If there is not green arrow, and a proxy server is involved, there may be an additional step required to configure the proxy. Instructions how to configure the proxy are at the end of the Lab. (Task 5)

- Copy and paste binaries onto the target

Select binary file and the source file from project
Press Ctrl-C (or right-click, Copy)
 Click on Remote Systems tab
Navigate through the ftp Files tree to desired path
Press Ctrl-V (or right-click, Paste)

- Executing binary on target from CCS RSE Terminal window

1.  From the Remote Systems tab, right-click Ssh Terminals
      Select Launch Terminal
2.  For newly transferred files, give them executable
      permissions:
	chmod 777 ./myHello

3.  Execute application
	./myHello

## Task 4: Optional – Use gdb to debug the application

In the edit prospective, select the project myHello

Select RunDebug configurations from file menu

Select C/C++ Remote Application

Click New button

Main tab:

  - Connection: select EVM [this is configured in WS1]

  - Specify Remote Absolute File Path for C/C++ Application (for example /bin/myHello if you pasted myHello in the bin directory)

.    Debugger  tab:

Main sub-tab:

  - GDB debuggerBrowse…:
 opt/gcc-linaro-arm-linux-gnueabihf-4.7-2013.03-20130313_linux/bin/arm-linux-gnueabihf-gdb

Shared Libraries sub-tab:

  - Add…
opt/gcc-linaro-arm-linux-gnueabihf-4.7-2013.03-20130313_linux/arm-linux-gnueabihf/libc/lib/arm-linux-gnueabihf

Press Apply, Debug

Using the debug in the debug prospective:

Using the debugger – one step into program:

Using the debugger – gdb console

Selecting gdb from the Debug panel allows direct control of gdb

Select the gdb and from the consol you can use almost all gdb commands, breakpoint, continue and so on.

Note – I was unable to move the pc backwards, or do reset or reload.  If I want to go backwards I had to close the remote debug terminal and open it again. Obviously the r (run) command does not work (run can start execution from the beginning of the execution)

## Task 5: Configure the host proxy

CCS defaults to using the system proxy settings (should not be needed if the target and the host are on the same subnet).

Right-click on the connection, select Properties

From the left pane, select Host.  Click Configure proxy settings

From Network Connections, set Active Provider to Direct, click
Apply and OK